University of Lagos student course

# Quiver student
laboratory handbook

Daily practice  •  Equipment induction  •  Parallel build work

Reference aircraft from the project repository. Remove all propellers for classroom and bench work.

## Learn the routine then build

This course teaches students to work safely, care for shared equipment and complete a supervised Quiver build. Start with daily laboratory practice, complete the common induction, then join one of six build teams.

For each activity, read the procedure, watch the instructor demonstrate, practise under supervision and show the required evidence. Use the opening and closing checklist at every session.

## Course outcomes

Identify hazards and isolate equipment; manage tools and components; use authorised lab equipment; explain Quiver systems; contribute a checked subsystem; run and evaluate a simulation; and hand over traceable work.

Local adoption is required before practical teaching. The department completes the room arrangements and approves equipment procedures. Course completion, equipment authorisation and real flight approval are separate decisions.

# Find a procedure

Follow the course in order during induction. Later, use this guide or the website search to find the activity you are about to perform.

## Daily lab practice

Daily laboratory rules

Access clothing and responsibilities

Opening the bench and isolating equipment

Components tools and labelled storage

Handling Quiver's TATTU 4.0 battery

Charging with the TATTU TA7200Pro

Computers files and power interruptions

Electrical benches and test instruments

Hand tools soldering and other apparatus

Printing a Quiver part on the Bambu Lab P2S

Handling the aircraft and changing payloads

Damaged equipment and emergency action

Returning items and closing the session

## Build and simulate

Understanding Quiver systems

ArduPilot simulation practice

Organising parallel build work

M1 Structure

M2 Powertrain

M3 Power and wiring

M4 Avionics

M5 Payloads and enclosure

M6 Software and simulation

Integration checks and demonstration

## Checklists and records

Induction checks and course completion

Individual knowledge check

Daily opening and closing checklist

Issue return and equipment authorisation

Technical work fault and handover records

## Teaching and sources

Course timetable and instructor preparation

Sources and adaptation notes

Quiver project sources and course maintenance

Equipment manuals and demonstration kit

EVERY DAY

# Daily laboratory rules

These rules apply throughout every supervised Quiver session. A clean bench and a correct return are part of the work, not an optional extra.

## The rules students follow

1.  Sign in, identify your supervisor and confirm your permitted task. Do not work alone on batteries, powered hardware or workshop equipment.

2.  Inspect your bench and equipment before use. Report damage or missing items before starting.

3.  Remove every propeller for bench work. Disarming does not remove electrical energy.

4.  Isolate all supplies before changing wiring, tools or payloads. Include USB and test supplies.

5.  Use only equipment for which you have recorded authorisation. Keep guards and protection in place.

6.  Take components only from the assigned, labelled storage. Record controlled tools and parts issued to you.

7.  Keep small parts in trays. Keep usable, unfinished and rejected items separate.

8.  Keep food, drinks, bags and loose objects away from work surfaces. Leave exits and walkways clear.

9.  Follow the battery procedure. Only the authorised electrical supervisor controls charging, connection and first power-up.

10.  Stop for smoke, unexpected heat, unusual movement, a damaged cable or an unclear instruction.

11.  Report faults, injuries and near misses promptly. Never return a defective item to usable stock.

12.  Before leaving, return and account for items, clean the bench, save files and complete the shutdown or handover.

## What good practice looks like

You finish a wiring task with two spare connectors. Check their part numbers, confirm they are clean and undamaged, return them to the correct bins and update the issue record. Put an unidentified connector in the labelled review tray and tell the stores lead. Do not guess its bin.

## Practise and show

Prepare a bench with a parts tray, an approved job sheet and only the tools needed. Explain how you would deal with a missing tool and a damaged connector.

Pass evidence: the instructor sees correct preparation and a complete return and reset. See sources S1, S2 and S3 in References.

EVERY DAY

# Access clothing and responsibilities

Your outcome: enter the laboratory prepared and recognise the limits of your permission.

## Before entering

•  Attend the room induction. Sign in using the laboratory's approved record and sign out when leaving.

•  Know who is supervising your session. Visitors need permission and an escort.

•  Wear closed footwear and clothing suited to the task. Secure long hair, loose sleeves and jewellery.

•  Use the eye protection, extraction and other protection specified for your task. Ask for properly fitting equipment.

•  Tell the supervisor if fatigue, illness, medication effects or an access need may affect safe work. Arrange support before starting.

## What your course pass permits

A general induction permits only the activities specifically recorded by your assessor. Separate authorisation is required for soldering, battery work, powered tests and each printer or machine. Never use another student's authorisation.

Students may inspect, organise, assemble or configure the tasks released to their team. The electrical supervisor controls battery connection and first power-up. The integration lead controls work on the shared aircraft.

## Conduct around others

Keep conversation and movement calm around active work. Do not distract someone soldering or operating equipment. Ask before touching another team's assembly or changing a shared computer configuration. Anyone may call STOP; pause and let the supervisor resolve the concern.

Keep emergency doors, switches, alarms and access routes unobstructed. Never rearrange benches, cabinets or fixed equipment without approval.

## Find these places during induction

Exit and assembly point; first-aid contact; approved isolation controls; assigned cabinet; battery storage and charging area; fault reporting point; printer stop control; waste collection points.

Local contacts and locations must be posted before the session begins. Use the actual university arrangements, not telephone numbers copied from overseas sources.

## Practise and show

Walk the exit route and identify your assigned work area and storage. State one activity you may perform and one that requires further authorisation. The assessor records the result. Sources S1, S3 and S5.

EVERY DAY

# Opening the bench and isolating equipment

Your outcome: check the work area and establish a safe state before touching hardware.

## Open the session

1.  Read the job sheet and last handover. Confirm the assembly revision, task owner and outstanding defects.

2.  Check the bench is dry, stable and clear. Put bags in the assigned location.

3.  Inspect tools, plugs, leads and connectors. Report missing guards, exposed conductors or damage.

4.  Prepare only the required items. Use a tray for fasteners and a suitable support for the assembly.

5.  Identify every possible energy source. Confirm that propellers have been removed.

6.  Agree the test or assembly method with the supervisor before starting.

## Isolate before changing anything

1.  Announce that work is stopping. The authorised operator stops the equipment using its approved procedure.

2.  Identify and disconnect each supply that can energise the work: flight battery, USB, bench supply, companion-computer supply and any other feed.

3.  Prevent reconnection using the laboratory's isolation and tagging procedure. Label the assembly's state.

4.  Follow the specified discharge time. A competent person verifies safe conditions at the approved test points.

5.  Start work only when the supervisor releases the isolated assembly.

Never short a capacitor or terminals to discharge energy. A switched-off device may still be connected to a live supply. A disconnected battery still stores energy.

## Restart deliberately

Account for tools and loose parts. Check that cables, guards and connections are correct and everyone is clear. Only the authorised person removes the isolation tag and approves reconnection. Other teams must not alter the shared aircraft during a powered test.

## Practise and show

On de-energised training equipment, identify all supply paths, attach the correct status label and explain how reconnection is prevented. The instructor assesses the sequence without asking students to work on an exposed live circuit.

Stop if: the supply path, isolation status or required test point is uncertain. Record the issue instead of assuming that a dark indicator means the equipment is safe.

EVERY DAY

# Components tools and labelled storage

Your outcome: take, track and return items without mixing parts or hiding defects.

## Take an item

1.  Read the cabinet and bin labels. Use only storage assigned to your team.

2.  Match the part number, size, rating and revision to the job sheet. Look at the container label, not only the appearance.

3.  Check condition and quantity. Record controlled items before removing them: item ID, quantity, student, team, date and destination bench.

4.  Put small components in a labelled tray or suitable protective packaging. Keep boards in approved ESD packaging.

5.  Close drawers and cabinets. Do not leave parts on the floor or in walkways.

## Keep three states separate

•  Usable stock: identified, accepted items in their correct bins.

•  Work in progress: an assembly or part awaiting work or inspection, labelled with team, owner, revision, status and date.

•  Quarantine or review: damaged, suspect or unidentified items held outside usable stock.

The diagram shows an example layout. The lab manager assigns the actual locations. Damaged batteries follow the specialist battery procedure; they do not go into the ordinary parts quarantine tray.

## Return an item

Remove debris using the approved method; inspect condition; count what remains; return accepted parts to the exact bin; update the issue record. Return leads to their rack and keep probes with their instrument. Report consumables used and tools or components that cannot be accounted for.

Do not mix similar screws, connector variants or component ratings. Do not silently substitute another item. Put uncertain parts in review and ask the stores lead.

## Practise and show

Sort 2111 side attachment spacers and 2131 bottom spacers into their own labelled bins. Keep 3112 CW and 3122 CCW propellers separately labelled and off the aircraft. Record one issue/return and route an unidentified spacer to review.

Pass evidence: correct identification, traceable quantity and correct storage location. Sources S1, S2 and S3.

EQUIPMENT

# Handling Quiver's TATTU 4.0 battery

Your outcome: recognise a suspect battery, protect its terminals and follow the staff-controlled handling procedure.

## Before handling

The course example is the TATTU 4.0 HV 53.2 V, 14S, 20,000 mAh, 25C LiPo smart battery selected for Quiver. Read the actual label and record its asset ID; do not substitute the 30 Ah pack also listed in the project BOM.

Read the label: 14S means fourteen cells in series; 20,000 mAh is 20 Ah. The nominal energy is 1,064 Wh. 53.2 V is nominal voltage, not a charging target; 25C is a discharge rating, not a charger setting.

Inspect visually without squeezing, dismantling or probing the pack. Look for swelling, cuts, punctures, dents, leakage, unusual heat, damaged insulation and damaged connectors. Report a dropped pack or hard landing even if the outside looks normal.

## Move and use an accepted pack

•  Use the TATTU carrying handle and the lab's approved handling method. Obtain help for its weight; never lift it by its connector.

•  Protect exposed terminals against contact with tools, jewellery and other conductive objects.

•  Keep the pack secure and away from heat, moisture and crushing loads.

•  Identify Quiver's 2211 battery slider and approved retention. Check fit and clearance with the pack switched off, under supervision.

•  Only the authorised person connects or disconnects the flight battery under the agreed procedure.

## If a pack is suspect

Stop using it and notify the electrical supervisor. Do not charge it, reconnect it to test it, open it or attempt a repair. Keep people clear. Staff decide whether and how it can be safely isolated and moved to the designated battery quarantine location.

If it hisses, smokes, vents or rapidly heats, raise the alarm and withdraw under the emergency plan. Do not carry an actively failing battery to a cabinet or attempt firefighting.

## Practise and show

On an unconnected, instructor-released TATTU pack, point to the label, handle, connector and indicators. Explain that an unlit indicator does not prove electrical isolation. Use photographs for damaged-pack examples.

Record: pack ID, observed condition, event or impact, date, reporter and supervisor decision. A student's visual check is not an independent declaration that a pack is safe.

Sources S4, S5 and E1. Follow the actual pack manual and laboratory procedure.

EQUIPMENT

# Charging with the TATTU TA7200Pro

Your outcome: understand the charging and storage controls and recognise when the session must stop.

## Before charging

Demonstration pair: the TATTU 4.0 14S 20 Ah smart battery and TATTU TA7200Pro, a charger listed in its manual. Staff verify the actual pack variant, firmware and matching manufacturer lead. ACES and Molex variants are not interchangeable; never force or improvise an adapter.

The pack manual specifies a 60.2 V charging upper limit (4.3 V per cell). This is a verification limit, not an instruction to configure a generic charger. Do not apply a generic 4.35 V-per-cell LiHV profile.

Use the designated noncombustible, ventilated charging area and approved containment. Keep at least 30 cm between charger and pack, and between packs; staff may require more. Do not improvise a sealed box.

Facilities staff must approve the TA7200Pro supply circuit and cable. Do not power it from a computer extension strip. Record pack ID, charger ID, operator, approved mode and start time.

## While charging

•  A trained authorised person supervises throughout the charge cycle. Do not leave charging unattended.

•  Observe the TATTU communication indicator, voltage, current and temperature. Do not select Fast mode to shorten a class.

•  Report any alarm, unexpected heating, swelling, smell or damaged connection.

•  For a normal stop, staff hold the Pause/Stop control as the manual directs and verify charging has stopped before disconnecting. For smoke, venting or fire, raise the alarm and withdraw; do not approach to unplug.

•  Never change charger settings merely to clear a fault.

## Return to storage

Staff apply the TATTU manual's storage and battery-management procedure, record the storage state and set the next inspection date. Do not guess a storage voltage or bypass the smart-battery controls.

Protect terminals individually; separate packs so connections cannot short; record their condition and location. Store only in the approved battery location and containment, away from ordinary components and combustible stock. Damaged packs require separate staff-managed arrangements.

Never put batteries in general rubbish. Staff arrange disposal through the approved route.

## Practise and show

First rehearse with the TA7200Pro disconnected. Identify channel A/B, screen and Pause/Stop control. Charging can start automatically about 14 seconds after successful battery communication. Staff must complete checks before connecting; students then read and log the display during the supervised demonstration.

Check: a charge continuing past class requires an authorised handover or safe stop. Sources S4, S5, E1 and E2.

EQUIPMENT

# Computers files and power interruptions

Your outcome: use the shared workstation responsibly and leave files and equipment ready for the next student.

## Begin at the workstation

Inspect the desk, monitor, keyboard, cables and power connections. Report damage, overheating or loose outlets. Keep liquids, food, batteries and soldering work away from computer desks.

Adjust the chair and screen for a comfortable position and take short movement breaks. Keep ventilation openings clear. Use your assigned account and approved software; do not share passwords or disable security controls.

## Protect the work

•  Save files in the approved team location. Use meaningful names containing the part or task and revision.

•  Keep a dated working copy and use the agreed backup or version-control method.

•  Record changes to CAD, code and parameters. Preserve the approved baseline.

•  Do not delete, rename or overwrite another team's work without agreement.

•  Do not connect unknown USB devices or install unapproved software. Ask before connecting equipment to the university network.

•  Keep passwords, keys and personal student records out of public repositories.

## If power or network service fails

Save work if the system permits. Follow staff instructions for orderly shutdown. A UPS can remain energised after mains power fails; do not open it or repair its wiring.

Do not repeatedly switch equipment on and off. Staff inspect and release affected printers, supplies and other equipment before restart. Check restored files against the last saved revision. Report a failed backup or corrupted file.

## Before leaving

Save and back up the work, record its location, close the authorised session and lock or log out. Return shared peripherals and report faults. Do not switch off a shared circuit that supports an active authorised job.

## Practise and show

Save the 2313 cover's Bambu Studio .3mf training project in the team folder, with its drawing revision and student identifier. Retrieve the backup on the approved workstation. For SITL, save the launch command, parameter file and flight log together. Lock the workstation and explain the power-failure response.

Pass evidence: saved and retrievable work, protected account and a clear handover. Local university ICT rules take precedence over examples from other institutions.

EQUIPMENT

# Electrical benches and test instruments

Your outcome: prepare an approved measurement without creating a short circuit or exposing others to energy.

## Before a measurement

Identify the quantity, expected range, test points and procedure. Inspect the instrument, leads and probes. The supervisor checks the circuit and setup before power is enabled.

Use only the permitted low-energy training circuit until authorised for other work. No student mains work, UPS repair or improvised high-current testing is included in this course.

## Use the correct instrument method

| Instrument | Required practice |
| --- | --- |
| Multimeter | Confirm sockets, function and range before contact. Use resistance or continuity only on isolated circuits. Never place a current-mode meter directly across a battery or supply. |
| Bench supply | The supervisor approves voltage, polarity and current limit. Keep the output off while making or changing connections. |
| Oscilloscope | Confirm the ground-reference arrangement before connecting. A ground clip can short a circuit. Use approved points; never defeat protective earth or probe mains. |
| ESD equipment | Use the approved setup for isolated electronics. A wrist strap prevents some static damage; it is not protection from electric shock. |

## Record and return

Write the expected and observed result with units, instrument identity and circuit conditions. Stop for a reading outside the approved acceptance criteria. Do not adjust a result or bypass protection to obtain a pass.

After the authorised shutdown, remove connections safely, return leads to their racks and keep probes with the instrument. Leave controls and leads in the state specified by the lab procedure.

## Practise and show

Use an isolated HAR-0008 front-right ESC signal harness as the training item. Confirm its connector-view drawing, select the correct meter sockets and continuity setting, and record a point-to-point check. Never place this meter setup across the TATTU battery.

Stop if: leads are damaged, the circuit state is uncertain, the expected range is unknown or the instrument is unsuitable. Sources S2 and S3 support inspection, authorised use and proper return.

EQUIPMENT

# Hand tools soldering and other apparatus

Your outcome: choose the correct tool, use the authorised method and return it in a safe condition.

## Hand tools

Check handles, jaws, cutting edges and insulation. Use the correct size and type; do not improvise a screwdriver as a lever. Secure the work and keep fingers clear of cutting and pinch points. Collect cut wire ends and fasteners so they cannot enter the aircraft.

Use only documented torque and adhesive requirements. If a value or curing time is missing, stop and obtain the engineering owner's instruction. Do not invent a setting.

## Soldering

Separate authorisation is required. Use the approved local fume extraction, eye protection, stable iron stand and work support. Work on isolated circuits. Keep the hot tip and lead clear of cables and people; never leave a hot iron loose on the bench.

The instructor demonstrates a sound training joint and the inspection method. Allow joints to cool without disturbance. Wash hands after handling solder and workshop materials. Collect solder waste through the approved route.

## Chemicals and cleaning

Read the label and safety data sheet before using an adhesive, solvent or cleaner. Use the specified ventilation and protection. Keep containers labelled and closed. Do not decant chemicals into drink bottles or combine unknown substances.

## Specialist equipment

Drills, saws, laser cutters, CNC machines, resin printers and composite cutting or sanding require separate training, controls and authorisation. General induction does not permit their use. Dusty fabrication belongs in its assigned area, away from computers and electronics.

## Return and report

Clean tools by the approved method, inspect them and return them to the labelled location. Tag and report faults; do not return a defective tool to the ordinary rack. Never remove guards or repair mains equipment.

## Practise and show

Select the documented driver for a 2313 PCB-cover fastener and support the isolated assembly. For crimp practice, use the terminals specified for HAR-0008 on a training sample. Solder only a separate practice coupon; do not practise on Quiver's flight boards. Return each tool to its labelled place. Sources S1, S3 and P3.

EQUIPMENT

# Printing a Quiver part on the Bambu Lab P2S

Your outcome: prepare an approved job, observe it safely and inspect the finished part.

Training example: Quiver 2313 battery connector PCB cover. Project image; inspect the approved CAD revision before printing.

## Before pressing print

1.  Identify the Bambu Lab P2S, its pause/stop controls and trained supervisor. Check cables, build area and enclosure.

2.  Confirm the approved ventilation and material. Air conditioning alone does not establish control of printing emissions.

3.  Open the released 2313 model in Bambu Studio. Select P2S and the installed nozzle, build plate and filament. The BOM specifies PETG-CF; staff must verify material and nozzle compatibility.

4.  Check orientation, supports, build-volume fit and the sliced Preview. Save a .3mf job with the part revision and profile. Staff approve it before printing. Never scale a part merely to fit.

## During the job

Watch the first layer. Keep hands clear of the P2S toolhead, heated bed and moving mechanisms. For poor adhesion or a filament tangle, pause/stop and call the supervisor; do not reach inside during motion or defeat interlocks. Nozzle changes require separate training.

For smoke or fire, raise the alarm and withdraw. Do not open the enclosure to investigate. An unattended job requires the laboratory's explicit written arrangement.

## After the job

Allow the bed and part to cool as directed by the P2S manual. Remove the plate/part using its approved method. Inspect the 2313 mounting holes, edges, warping and layer bonding against the drawing.

Label accepted parts with identity and revision; put rejected parts in review. Appearance alone cannot prove flight suitability. After a power interruption, a technician releases the printer and the instructor reviews any resumed structural print.

## Practise and show

Prepare a 2313 training copy, demonstrate safe removal and measure one specified feature. Label it TRAINING until formally accepted. Log the result, return tools and reset Bambu Studio for the next user.

Sources S6, E3 and P2. AMS is optional: use the approved feed path actually installed.

EQUIPMENT

# Handling the aircraft and changing payloads

Your outcome: move and work on the assembly without exposing people to rotating parts, trapped fingers or unintended power.

## Before moving the aircraft

Identify its weight, centre of gravity and approved lifting points. Clear the route and prepare a stable stand. Use the required number of people or lifting aid; do not lift by propellers, cables, sensors or unsupported covers.

The supervisor confirms isolation. Remove and account for every propeller before bench work. Support the aircraft so it cannot tip or fold unexpectedly.

## Folding arms and mechanical work

Inspect joints and latches using the approved drawing and method. Keep fingers clear of hinges and pinch points. Check cables through the permitted movement range. Never force a misaligned joint or alter a structural part to make it fit.

Protect openings against loose fasteners and wire offcuts. Count tools before closing covers. Record incomplete fasteners, curing joints and restrictions on the handover label.

## Payload changes

1.  Obtain the integration lead's release and establish safe isolation.

2.  Support the payload before releasing the latch or connector.

3.  Identify the approved attachment, power and data interfaces.

4.  Fit the inert training payload and inspect retention, clearance and cable strain relief.

5.  Record its mass, mounting position and outstanding checks.

This course requires isolation for every swap. A quick-release interface does not establish that live electrical connection is permitted. Fit and retention alone do not prove a payload is approved for flight.

## Powered checks

Only the integration lead's authorised operator energises the shared aircraft. Other teams remain clear and do not change wiring or parameters. Follow the written stop plan. If a motor moves unexpectedly, call STOP and step clear; do not grab it.

## Practise and show

On isolated Quiver, identify the 1411 folding-arm connector and its pinch points. At the bottom attachment, identify the 2131 spacer, 2112 quick-release plate and HAR-0014 bottom-payload harness. Fit an inert training payload; record mass, retention and clearance without energising the interface.

EVERY DAY

# Damaged equipment and emergency action

Your outcome: stop unsafe work, prevent reuse of defective equipment and give a useful report.

## An ordinary fault without immediate danger

1.  Stop the activity. Make the area safe only within your training and authority.

2.  Tell the supervisor promptly. Do not reconnect an item to see whether the fault repeats.

3.  Mark the item DO NOT USE with its asset ID, observed fault, date and reporter.

4.  The supervisor isolates it and assigns the correct quarantine or repair location.

5.  Record the event and its effect on other assemblies. Only authorised staff release it back to service.

Do not return suspect tools, boards or cables to usable stock. Do not place a damaged battery in the ordinary equipment quarantine area.

## Smoke venting or fire

Raise the alarm, withdraw and evacuate by the safe route. Go to the assembly point and follow staff instructions. Do not carry burning or venting batteries, open an affected enclosure, attempt firefighting or re-enter to retrieve equipment.

## Injury or electric shock

Alert staff immediately. Keep clear of electrical hazards. Do not touch a person still in contact with an electrical source. Trained responders isolate the danger and provide first aid. Use the posted university emergency process.

## Report near misses too

Report a dropped pack, unexpected motor movement, wrong connector, overheated lead or missing guard even when no injury occurred. Describe what you observed, what was connected, who was informed and what remains isolated. Do not assign blame or conceal an error.

## Practise and show

Use a safe scenario: a frayed lead is found before use. Demonstrate the stop, label, report and segregation sequence. Then explain how the response changes if a battery is actively smoking.

Local details required: campus emergency contact, room location, alarm method, exit, assembly point and first aider. The instructor must display these before work.

Sources S3 and S4 inform reporting and battery response. Overseas emergency numbers and site-specific fire procedures are not adopted here.

EVERY DAY

# Returning items and closing the session

Your outcome: leave a traceable, safe work area and an unambiguous handover.

## Close in this order

1.  Stop work in time. Save the final task state and identify incomplete work before the session ends.

2.  Shut down or hand over. The authorised operator follows the equipment procedure. Do not interrupt a shared supply, required extraction or active job without agreement.

3.  Isolate the assembly. Label the energy state and outstanding hazards. Staff control flight-battery handling.

4.  Count and return. Account for tools, components, leads and consumables against the issue record.

5.  Inspect and separate. Return only accepted items to usable storage. Label unfinished work; report and segregate suspect items.

6.  Clean and reset. Collect fasteners, wire ends and approved waste. Use the correct cleaning method; do not blow hazardous dust around the room.

7.  Save and report. Back up files, record revisions and tell the next responsible person what is complete and restricted.

8.  Obtain the close check. The assigned checker or supervisor inspects the bench. Sign out.

## A useful handover says

Assembly and revision; owner and next operator; work completed; evidence location; current power and isolation state; open faults; items missing; next permitted task; person accepting the handover.

“Finished” or “should be okay” is not enough. If a tool is missing, report it before the aircraft is closed or released. If a job continues, name the authorised person taking responsibility.

## Example

The frame is assembled but adhesive is curing. Label the assembly with the product and procedure reference, recorded application time and the specified release time. State that loading or movement is restricted until the supervisor releases it.

## Practise and show

Reset a training bench containing spare parts, a suspect cable and unfinished work. Complete the issue/return record and explain the handover to another student.

Pass evidence: complete inventory, correct segregation, an accepted bench and a clear record. Sources S1, S2 and S3.

BUILD

# Understanding Quiver systems

Your outcome: identify the major subsystems and explain how power and commands reach the motors and payload.

Quiver combines an airframe, propulsion assemblies, custom interface and power boards, flight-control hardware, sensors and software. The course uses the documented project as a reference; the issued kit must be checked against the approved release.

## Recognise the components

•  Structure: frame, landing gear, arm attachments and supporting mounts.

•  Powertrain: motors, electronic speed controllers or integrated propulsion units, arm joints and the removed propellers.

•  Power and wiring: battery interfaces, distribution, protection, connectors and harnesses.

•  Avionics: flight controller, sensors and communication hardware.

•  Payloads: bottom and side attachment hardware, enclosure and associated electrical/data interfaces.

•  Software: flight firmware, parameters, ground software and simulation.

The Pix32 V6 is a flight-controller module, not a single chip. ArduPilot is firmware that runs on compatible flight-control hardware; in SITL, flight-control software runs on a computer with a simulated vehicle.

## Find the right project file

Use the selected BOM and manufacturing drawings to identify parts. Use harness drawings and board documentation for connector identity and pinout. Use firmware documentation only after the physical board and firmware target have been verified.

The reference repository includes attachment-interface CAD under src/quiver/supporting_structure/attachment_interface. A drawing image is not a load rating or an approved manufacturing specification.

## Practise and show

On the isolated assembly or labelled photographs, point to each subsystem. Trace one power path and one control path. Identify where your team's work meets two other teams.

Evidence: a labelled system sketch and a short explanation of one interface. See project sources P1 to P5 in Project references.

BUILD

# ArduPilot simulation practice

Your outcome: run a repeatable flight-mode exercise and explain its limits.

SITL means software in the loop. It lets you practise with flight-controller software and a simulated aircraft. It can teach modes, telemetry and mission logic. It does not establish Quiver's real endurance, payload capacity or handling without a validated physical model.

## Prepare the session

The instructor provides a tested installation, launch instructions, vehicle model and parameter baseline. Record firmware version or commit, model, parameter-file revision and launch command.

Confirm that only simulator endpoints are connected. Disconnect real-vehicle connections and label the session SIMULATION. Do not upload settings to shared hardware during another team's work.

## Fly the common exercise

1.  Start the approved scenario. Check simulated position, home, mode and warnings.

2.  Follow the instructor's arm and take-off procedure.

3.  Hold position, make a short commanded move and observe altitude, speed and mode changes.

4.  Use the configured return procedure, then land.

5.  Save the log and a screenshot. Explain one unexpected result or warning.

6.  Repeat individually as operator while a partner observes.

Guided follows supplied commands. Loiter attempts to hold position and altitude when the required estimates are available. RTL follows configured return behaviour. Land descends according to the configured landing behaviour.

## Keep the result honest

The Quiver repository includes an obstacle-avoidance SITL evaluation with approximate aircraft configuration and simulated sensors. Its launch command alone is not a complete, validated Quiver physics model. Advanced exercises need the matching firmware, parameters and sensor configuration verified by the instructor.

Change only one approved parameter at a time. Record the baseline, prediction, change and result, then restore the baseline. Never bypass a safety check to obtain an apparent success.

## Practise and show

Submit your launch record, one flight log, one explained anomaly and two model limitations. Another student should be able to repeat the setup.

Sources S7 and P5. Passing the simulation exercise does not authorise real flight.

BUILD

# Organising parallel build work

Your outcome: know what your team owns and what must be agreed before it connects to another team's work.

## Six team responsibilities

| Team | Main work | Handover |
| --- | --- | --- |
| M1 Structure | Frame, landing gear and fixed mounts | Fit and mounting records |
| M2 Powertrain | Motor arms, joints and propulsion units | Inspected assemblies and motor map |
| M3 Power and wiring | Power interfaces and harnesses | Pinout, polarity and inspection evidence |
| M4 Avionics | Controller, sensors and communications | Hardware and port map |
| M5 Payloads and enclosure | Attachments, quick release and covers | Retention, mass and interface record |
| M6 Software and SITL | Simulation and configuration | Repeatable setup, logs and limitations |

## Agree the work before starting

Use teams of two to four where supervision permits. Rotate builder, checker and recorder. The checker must be a different person from the builder.

Record the approved revision, required parts, task sequence, hazards, acceptance criteria and interfaces. Sending and receiving teams agree dimensions, connector identity, pinout, power/data requirements and test method. Missing values go to the engineering owner.

Teams may work on isolated subsystems in parallel. The integration lead controls the shared aircraft. No team may connect a supply, change wiring or upload settings during another team's work.

## Daily handover

State what was completed, where the evidence is saved, current isolation state, open defects and the next permitted activity. Keep individual evidence as well as the team record.

Before building: the instructor accepts the work plan and the applicable equipment authorisations. Open the work card for your team.

BUILD MODULES

# M1 Structure

Deliverable: a correctly assembled frame with secure, accessible mounting points.

## Prepare

Identify 1111 upper plate, 1112 middle plate, 1113 lower plate and 1310/1320 landing-gear tubes against the released drawings. Collect the fastener list, work support and inspection tools. M1 owns the fixed structure; coordinate arm and payload interfaces with M2 and M5.

## Work sequence

1.  Inspect parts for damage and check the required dimensions.

2.  Lay out fasteners in labelled trays. Record quantities issued.

3.  Assemble in the specified order using the documented fasteners, torque, adhesive and curing requirements.

4.  Check alignment, retention, clearance and cable routes.

5.  Invite receiving teams to check their interfaces before access becomes restricted.

6.  Record the assembly state and any limits on moving or loading it.

## Stop and ask

Stop for a drawing mismatch, damaged member, missing value or forced fit. Do not drill, thin, bend or cut structural parts without engineering approval. Composite work needs separate dust controls and training.

## Acceptance checklist

☐  Parts and revisions match the approved work package.

☐  Fasteners and joints meet the documented assembly and inspection method.

☐  Mounts, landing gear and cable routes are checked.

☐  M2, M4 and M5 have reviewed the applicable fit.

☐  Defects, curing restrictions and pending checks are recorded.

☐  Tools and spare components have been accounted for and returned.

## Show your understanding

Measure one selected interface and record required versus actual values with units. Explain how an incorrectly placed mounting hole affects the receiving team.

Submit: assembly photographs, part/revision list, inspection results and handover. The mechanical supervisor accepts the work. Each student provides one observed task and one record they produced or checked.

BUILD MODULES

# M2 Powertrain

Deliverable: identified propulsion assemblies with accepted mounts, joints and cable protection.

## Prepare

Identify the 3111 Hobbywing XRotor X6 Plus motor/ESC units, 1412 arm tubes and 1411 folding connectors. Compare labels with the approved motor-position map. Keep 3112 MFP 24×8.0 CW and 3122 CCW propellers removed, labelled and separately accounted for.

## Work sequence

1.  Inspect components and identify their aircraft positions.

2.  Check folding joints, locks and retention using the specified method.

3.  Fit motors and associated parts with documented fasteners and clearances.

4.  Route and protect cables against pinching, abrasion and tension.

5.  Check the permitted movement range on the isolated assembly.

6.  Label each unit and record required motor direction for later authorised checks.

## Stop and ask

Do not connect a battery or run motors at the team bench. Do not use generic ESC calibration instructions without verifying compatibility with the issued propulsion type. Stop for damaged wiring, uncertain orientation, a loose joint or a forced fit.

## Acceptance checklist

☐  Propellers absent and accounted for.

☐  Motor identity and position match the approved map.

☐  Mounts, joints and locking features inspected.

☐  Cables protected through the full permitted movement.

☐  M1 accepts fit; M3 and M4 accept power and control interfaces.

☐  Powered rotation checks are clearly marked pending until authorised.

## Show your understanding

Demonstrate the joint inspection and identify a possible pinch point. Explain why a secure mount does not prove correct motor configuration.

Submit: labelled motor map, mechanical inspection, photographs and pending-test list. The mechanical supervisor accepts the assembly; the integration lead schedules any powered checks. Close with the standard inventory and bench reset.

BUILD MODULES

# M3 Power and wiring

Deliverable: identified harnesses and power connections with independent inspection and test evidence.

## Prepare

Identify 3310 Main PCB, 3320 Battery Connector PCB and the released harness drawings. Use HAR-0008 ESC signal FR for the continuity demonstration. Obtain the specified wire, terminals, crimp tool and instruments. Confirm pin numbering and connector viewing direction; the TATTU pack stays disconnected.

## Work sequence

1.  Match connector and wire identities to the approved drawing. Never infer function from colour alone.

2.  Label both ends as specified. Keep unused terminals and offcuts separate.

3.  Assemble or inspect only the harnesses released to your team.

4.  Check crimps, insulation, retention, strain relief, routing and protection.

5.  On isolated equipment, perform the approved point-to-point, polarity and unintended-connection checks.

6.  Record expected and actual results with units and instrument identity.

7.  Obtain independent electrical review before installation or power-up.

## Stop and ask

Stop for ambiguous pinouts, damaged insulation, wrong parts, a failed test or missing acceptance criteria. Do not improvise high-current joints, board repairs or test methods. The electrical supervisor controls battery connection and first power-up.

## Acceptance checklist

☐  Pin identity and polarity match the drawing.

☐  Crimps, insulation, retention and cable routes are accepted.

☐  Test results are traceable to the harness revision.

☐  Receiving teams have confirmed their supply and connector requirements.

☐  No unresolved electrical defect remains before release to integration.

☐  Scrap and suspect parts are separated from usable stock.

## Show your understanding

Demonstrate a check on an unpowered training harness. Explain how reversing the drawing's viewing direction could produce an incorrect connection.

Submit: connector schedule, inspection/test record, photographs and rework history. The electrical supervisor records acceptance; a student signature alone does not release the assembly for energisation.

BUILD MODULES

# M4 Avionics

Deliverable: installed flight-control, sensor and communication hardware with a verified port and configuration record.

## Prepare

Identify the 3332 Holybro Pix32 V6, 3330 flight-controller PCB and 3312 Raspberry Pi 5 using physical labels and the approved port map. Pix32 V6 is the flight-controller board; the Raspberry Pi is the companion computer. Verify the actual hardware before selecting firmware.

## Work sequence

1.  Match controller, carrier and sensor identities to the approved hardware record.

2.  Confirm orientation, mounting method, clearances and cable routes.

3.  Fit and label the authorised connections while isolated.

4.  Review every power source with M3, including USB and companion-computer feeds.

5.  Agree the firmware target, version and parameter baseline with M6.

6.  List required calibrations, communication checks and integration conditions.

## Stop and ask

Stop for an uncertain board target, incorrect orientation, damaged port or conflicting pinout. Obtain approval for each power arrangement. USB may energise part of a circuit. Do not flash firmware or change settings while another team is working on the shared aircraft.

## Acceptance checklist

☐  Actual hardware and selected firmware target match.

☐  Orientation and mounting are documented.

☐  Power and signal connections have independent checks.

☐  M3 and M6 accept the interfaces.

☐  Calibrations and powered checks are completed at the approved stage or explicitly listed as pending.

☐  Boards and sensitive components are stored in approved protective packaging.

## Show your understanding

Point out the Pix32 V6 orientation and trace one approved sensor connection. Inspect the 2311 Main PCB mount and specified vibration isolation. Explain why a displaced mount or changed assembly requires review before calibration.

Submit: port map, hardware identities, mounting photographs and configuration record. The avionics supervisor accepts the installation. Return probes with their instruments and record any loose or suspect connector before closing the session.

BUILD MODULES

# M5 Payloads and enclosure

Deliverable: secure payload attachment and enclosure assemblies with clear mechanical, power and data interfaces.

## Prepare

Collect 2111 side spacers, 2131 bottom spacers, 2112 quick-release plates, 3331 Attachment Interface PCBs and 2411/2412 enclosure parts, plus an inert training payload. Check the released CAD/BOM revision. M3 verifies power/data pinouts; attachment positions are not automatically electrically interchangeable.

## Work sequence

1.  Identify the three attachment locations and the parts assigned to each.

2.  Inspect manufactured parts using the approved dimensions and acceptance criteria.

3.  Assemble spacers, plates, latches and enclosure parts in the specified order.

4.  Fit the inert payload while the aircraft is isolated and properly supported.

5.  Inspect engagement, retention, clearance, cable routing and strain relief.

6.  Record payload mass and position. Agree supply and data requirements with M3, M4 and M6.

## Stop and ask

Stop for damaged prints, unclear load limits, inadequate retention or conflicting interfaces. Never infer a load rating from appearance or successful fit. Do not repair a structural print or conduct a load test without a separate approved method. All payload swaps require isolation.

## Acceptance checklist

☐  Correct parts and revisions fitted.

☐  Retention and clearance satisfy the approved inspection method.

☐  No unapproved structural modification or repair.

☐  Payload mass and mounting position recorded.

☐  Mechanical, electrical and data interfaces accepted by receiving teams.

☐  Any remaining payload flight approval is explicitly outstanding.

## Show your understanding

Demonstrate latch inspection with an inert payload. Explain why the location of the payload matters as well as its mass.

Submit: fit and retention evidence, part inspection, mass/location and interface record. The supervisor accepts the build; attachment acceptance alone does not approve a flight with that payload.

BUILD MODULES

# M6 Software and simulation

Deliverable: a repeatable simulator setup and traceable software configuration.

## Prepare

Use the approved ArduPilot SITL workstation, source revision, model and parameter files. Keep a separate Pix32 V6 hardware configuration record for M4; simulator settings are not automatically suitable for Quiver hardware. Confirm the team storage location and preserve the issued baseline.

## Work sequence

1.  Record software versions, source commits, launch commands and file origins.

2.  Verify that the session connects only to the simulator.

3.  Run the common take-off, hold, movement, return and landing exercise.

4.  Save logs and record warnings, model assumptions and recovery actions.

5.  Change one approved variable for an extension experiment, then restore the baseline.

6.  Write instructions another student can follow on a second approved workstation.

7.  Review any real-hardware configuration record with M4 and the integration lead.

## Stop and ask

Stop for an unexpected real endpoint, incompatible firmware target, unexplained warning or missing baseline. Do not bypass checks or send commands to shared hardware during another team's work. Add Quiver Hub or companion-computer integration only within the released scope.

## Acceptance checklist

☐  Versions, model, parameters and launch method recorded.

☐  Another student can reproduce the common exercise.

☐  Logs and model limitations are included.

☐  Changes and restored baseline are traceable.

☐  M4 accepts any hardware configuration interface.

☐  Files are backed up and credentials excluded.

## Show your understanding

Explain one observed behaviour using the log and distinguish a control setting from an assumption about the aircraft's physics.

Submit: setup instructions, configuration files, experiment record and logs. Each student must also operate the common exercise. The instructor accepts reproducibility; a successful simulated flight does not establish real flight safety.

BUILD

# Integration checks and demonstration

Your outcome: combine accepted subsystems through controlled checks, with one person responsible for the shared aircraft.

## Four checkpoints

| Checkpoint | Evidence required | Release |
| --- | --- | --- |
| 1 Ready to build | Induction, work plan, approved revisions and interfaces | Instructor |
| 2 Subsystem accepted | Inspection, test results, defect status and handover | Relevant technical supervisor |
| 3 Ready for powered checks | Unpowered integration, supply/polarity checks, propeller removal, tool count and written stop plan | Electrical supervisor and integration lead |
| 4 Demonstration complete | Authorised test records, SITL evidence, individual explanations and remaining restrictions | Course lead |

## Before energising

Confirm that no other team is changing the aircraft. Inspect mounts, cables, connectors, retention and clearances. Verify all required unpowered electrical checks. Confirm firmware target and approved configuration. Remove and account for all propellers and loose tools.

Identify the authorised operator, observers, exclusion area and stop method. Staff choose the appropriate supply and protection for the test. The test sheet must state expected results and stop criteria before power is applied.

## During and after a test

Run only the released test. Record conditions, observations and actual values with units. For an unexpected result, stop through the approved process, isolate, report and record. Do not repeat a failed powered test without supervisor review.

After rework, repeat the affected checks and record why they were selected. Keep the failed result in the record; do not replace it with a later pass.

## Demonstrate the learning

Each team shows its accepted subsystem, one inspection result and one interface. Each student explains their own contribution. Show the simulated flight and its limitations.

Boundary: this course ends with supervised bench and simulation evidence. Real flight requires a separate approved aircraft, site, crew, risk assessment and compliance with current Nigerian requirements. See S10.

RECORDS

# Induction checks and course completion

Your outcome: demonstrate the daily practices individually and know what the assessor has authorised.

## Induction is observed

For every assigned item, the instructor demonstrates the actual location or procedure, the student practises, and the assessor observes. A signature confirms recorded evidence; it does not replace the demonstration.

| Practical check | Student must show |
| --- | --- |
| Emergency orientation | Exit, assembly point, alarm and contact route |
| Opening and isolation | Supplies identified and safe training sequence explained |
| Stores | Correct issue, identification, return and review segregation |
| Battery awareness | Damage recognised, terminals protected and staff route explained |
| Workstation | Safe setup, saved file, backup and account lock |
| Assigned equipment | Required pre-use, operating and shutdown checks |
| Closing | Inventory, bench reset and accepted handover |
| Subsystem | One task completed and checked against criteria |
| Simulation | Common exercise, saved log and two limitations |

Record student, task/equipment, date, assessor, evidence, result and permitted activity. Mark unassigned equipment Not assigned; this gives no permission to operate it.

## Results

Pass: correct performance within required supervision and a clear explanation. Practise again: a specific gap needs instruction and reassessment. Stop: an unsafe condition or action requires intervention before continuing.

## Complete the knowledge check

Use the individual knowledge check. Explain each answer in your own words. The instructor can assess this orally where an agreed learning adjustment is needed and records the evidence.

Proposed completion rule: at least 8/10, all safety items 1, 3, 4, 5, 6 and 8 correct, and all applicable practical checks passed. The department adopts the final rule. Further practice and reassessment are recorded. Completed personal records belong in approved restricted university storage.

RECORDS

# Individual knowledge check

Answer in your own words. Your assessor uses this check alongside the practical demonstrations. Use a separate sheet if you need more space.

Student ____________________  Date ____________________

1. Why is disarming insufficient for wiring work?

Response ______________________________________________________________

2. Where does an unidentified component go?

Response ______________________________________________________________

3. What must happen before a damaged lead can return to service?

Response ______________________________________________________________

4. What is the response to a smoking battery?

Response ______________________________________________________________

5. May a charging pack be left unattended?

Response ______________________________________________________________

6. Why must every propeller be removed at the bench?

Response ______________________________________________________________

7. What do you record before handing over unfinished work?

Response ______________________________________________________________

8. Does a general induction authorise every machine?

Response ______________________________________________________________

9. Does SITL success prove exact Quiver handling?

Response ______________________________________________________________

10. What do you do if a required drawing value is missing?

Response ______________________________________________________________

Assessor ____________________  Score out of ten __________

Safety items 1 3 4 5 6 and 8 correct  Yes / Reassessment needed

Feedback and further practice _______________________________________

__________________________________________________________________

Record practical results separately. This sheet alone does not authorise equipment use or flight.

RECORDS

# Daily opening and closing checklist

Use a fresh record for each bench and session. The student completes the checks; the assigned checker or supervisor verifies the close.

Date and session ____________________  Bench ____________________

Team and students __________________________________________________

Supervisor ____________________  Job sheet and revision ____________

## Before starting

☐  Signed in and permitted task confirmed.

☐  Last handover and outstanding defects read.

☐  Exit and access routes clear; work area dry and stable.

☐  Clothing and task protection correct.

☐  Tools, leads and equipment inspected.

☐  Parts and controlled tools issued and recorded.

☐  Supplies and isolation state identified.

☐  All propellers removed before bench work.

☐  Supervisor has released the task.

Restriction or action required _______________________________________

__________________________________________________________________

## Before leaving

☐  Approved shutdown completed or named handover accepted.

☐  Aircraft/circuit isolation state labelled.

☐  Tools and components counted and returned to labelled locations.

☐  Suspect items reported and segregated through the correct procedure.

☐  Work in progress labelled with owner, revision and restriction.

☐  Bench cleaned; waste sorted; walkways clear.

☐  Files saved and backed up; account locked or logged out.

☐  Missing items and unfinished work reported.

☐  Checker or supervisor accepts the bench; students sign out.

Next permitted task __________________________________________________

Person accepting any continuing job ___________________________________

Student ____________________  Checker or supervisor _______________

Time completed ____________________

Do not sign a check you have not performed. If a condition fails, record it and obtain the supervisor's direction.

RECORDS

# Issue return and equipment authorisation

Use the lab's controlled register or this blank form. Keep completed student records in restricted storage.

## Issue and return record

Date ____________________  Team and bench ________________________

Student responsible _________________________________________________

Item ID and description ______________________________________________

Part number rating and revision _______________________________________

Storage location ____________________  Quantity issued ____________

Condition on issue __________________________________________________

Quantity returned __________  Used or consumed ____________________

Missing or quarantined quantity and reason ___________________________

__________________________________________________________________

Return location and condition ________________________________________

Returned by ____________________  Checked by _____________________

## Equipment authorisation record

Student ____________________  Equipment and asset ID ______________

Procedure and revision ______________________________________________

Demonstration and supervised practice date ___________________________

Observed task and evidence ___________________________________________

__________________________________________________________________

Result  Pass / Practise again / Stop

Permitted activity and required supervision __________________________

__________________________________________________________________

Restrictions and reassessment required _______________________________

Assessor and date ____________________________________________________

Authorisation is specific to the recorded equipment and activity. A change of machine, process or risk may require further instruction.

RECORDS

# Technical work fault and handover records

Use a separate copy for each event or test. Record facts, units and revisions so another person can understand the work.

## Inspection or test record

Assembly or job ____________________  Revision ____________________

Procedure and test points ____________________________________________

Conditions and instrument ID _________________________________________

Expected result and acceptance limits ________________________________

Actual result with units ____________________________________________

Pass or defect and action ____________________________________________

Builder ____________________  Independent checker _________________

Supervisor decision and date _________________________________________

## Fault or near miss record

Item and location ____________________  Date and time ______________

What was observed ____________________________________________________

What was connected or operating ______________________________________

Immediate action and current isolation state _________________________

Who was informed and when ____________________________________________

Quarantine location or restriction ___________________________________

Authorised release or follow-up owner ________________________________

## Handover record

Work completed and evidence location _________________________________

Open defects missing items and restrictions __________________________

Next permitted task and responsible person ___________________________

Handed over by ____________________  Accepted by ___________________

For a battery record also include pack/charger ID, approved settings, condition and operator. For a print include printer/material/profile, part revision and inspection. For SITL include firmware/model/parameters, launch command and log location.

INSTRUCTOR

# Course timetable and instructor preparation

Use this guide to adopt the course and prepare safe, achievable practical sessions. The rules become local operating requirements only through the responsible department and laboratory manager.

## Proposed timetable

| Session | Focus | Hours |
| --- | --- | --- |
| 1 | Room walk, daily rules, storage and isolation practice | 3 |
| 2 | Computers, battery awareness and assigned equipment induction | 3 |
| 3 | Systems and individual SITL practice | 3 |
| 4 | Team plans, drawings and interface agreements | 3 |
| 5 to 7 | Six supervised build teams | 9 |
| 8 | Unpowered integration and inspections | 3 |
| 9 | Authorised powered checks with propellers removed | 3 |
| 10 | Demonstrations, individual checks and feedback | 3 |

Thirty contact hours assume prepared kits and a tested simulator. Add time for fabrication, curing, rework or reassessment. Do not promise a flight-ready aircraft within this timetable.

## Complete before teaching

Name the course lead, laboratory manager, technical supervisors and stores lead. Set group sizes according to competent supervision. Inspect the rooms and equipment.

Post the actual emergency contacts, room identifier, exit, assembly point and first aider. Assign usable stock, work-in-progress and ordinary quarantine locations, plus separate battery arrangements. Prepare labels, issue records, safe training items and approved waste routes.

Approve drawings, pinouts, torque, adhesives, print specifications, battery procedures and software baseline. Resolve the lower-plate thickness conflict and the 20/30 Ah battery variants with the engineering owner. Prepare the TATTU 20 Ah/TA7200Pro demonstration and P2S/2313 job using the equipment references.

## Deliver each activity

Explain the outcome and hazards, demonstrate on the actual equipment, supervise practice, observe each student and record feedback and permission. Make reasonable learning/access adjustments while preserving the safety requirement. Revisit closing and storage habits at every session.

## Assessment notes

Answers: 1 connected energy remains; 2 review location; 3 authorised inspection/release; 4 alarm and withdraw; 5 no; 6 prevent rotating-part injury; 7 status, evidence, defects and next task; 8 no; 9 no; 10 stop and obtain the approved instruction.

Review incident patterns, missing items, feedback and timings after the cohort. Record improvements, owners and review dates. Sources S8 and S9 provide educational context; this course is not independently accredited.

REFERENCES

# Sources and adaptation notes

The course adapts practical habits from university laboratories and applies them to a supervised Quiver build. Local procedures and the actual equipment manuals determine authorisation and operating limits.

## Daily laboratory practice

S1 Maryland Robotics Drone Lab — Rules and safety. Used for assigned storage, authorised tools and returning equipment. Its local flight thresholds, emergency contacts and instructions to move burning equipment are not adopted.

S2 Fresno State — Electronics laboratory safety, operating guidelines 14 to 16. Used for component trays, lead racks, keeping probes with instruments and bench reset. Its local network rules are not UNILAG rules.

S3 Penn State Behrend — Laboratory and shop procedures. Used for documented equipment training, proper return and damaged-item reporting. Local access rules and penalties are not transferred.

S4 Queen Mary University of London — Research labs and drones. Used for supervised charging, inspection and protected storage. UK law references and emergency numbers are not Nigerian requirements.

S5 NUbots — Lab induction checklist, battery items 1.1 to 1.6 and signatures. It records instruction and acknowledgement. Our observed student demonstrations are an added Quiver course design choice. Its software ownership agreement is not adopted.

## Equipment and education

S6 NIOSH — Approaches to safe 3D printing. Emissions and controls for makerspaces and schools.

S7 ArduPilot — SITL documentation and motor setup safety. Verify the exact hardware and software before applying instructions.

S8 UNILAG — Student centred learning guideline and university policies.

S9 NUC — Engineering CCMAS. Programme context; the department determines any credit mapping.

S10 NCAA — Part 21 RPAS. Check current requirements before real flight operations.

## Design and provenance

Use short procedures, nearby diagrams, worked examples and repeated practice. CDC information chunking and IES instructional guidance inform this approach. They do not prescribe this course's assessment threshold.

Version 5.1 prepared 25 October 2026. The aircraft photo is an unaltered project asset; diagrams are conceptual teaching aids. Project technical references are listed on the next page.

REFERENCES

# Quiver project sources and course maintenance

## Project baseline

The engineering reference snapshot is commit 3ed4eb25c4fbef565b2f68804b499a328c56abe1 in the Arrow Air project repository. This identifies provenance; the engineering owner must approve the actual kit and procedures.

P1 Project overview — README.

P2 Parts and assembly — BOM and manufacturing guide.

P3 Wiring and boards — Harness guide, main PCB, battery-control PCB and flight-controller PCB.

P4 Firmware — Operations firmware reference.

P5 Simulation — Quiver SITL evaluation.

P6 Engineering report — Dev Kit Engineering Report. Its standard configuration uses 20 Ah; the BOM also lists 30 Ah. This course uses the requested TATTU 4.0 14S 20 Ah example. The engineering owner must reconcile the actual build configuration before procurement or use.

The repository identifies its licence as CERN-OHL-S v2. Preserve project attribution and review the applicable licence and any third-party asset rights when redistributing engineering material.

## Maintain the course

Keep the Markdown pages as the course source. Update the handbook and website together when a procedure, kit, room or software baseline changes. Record the change, responsible reviewer and effective date.

The website provides search, page navigation, print support and a handbook download. It is a documentation site, not a shared student-record database. Edit its Markdown files and rebuild to publish changes.

## Local adoption record

Department and laboratory ____________________________________________

Course owner _________________________________________________________

Technical and safety reviewers _______________________________________

Approved hardware and software baseline ______________________________

Adoption date ____________________  Next review date ______________

Complete these records in the approved administrative copy. Keep personal assessment, incident and authorisation records in restricted university storage.

REFERENCES

# Equipment manuals and demonstration kit

Use these references alongside the daily procedures. The instructor checks the manual supplied with each actual unit and records its revision before practical teaching.

## E1 TATTU 4.0 battery

Course example: HV 53.2 V, 14S, 20,000 mAh, 25C LiPo smart battery. The 20 Ah model in the manual is TA4TCG20K1425X. Check the physical label and connector variant.

TATTU 4.0 14S 20/30 Ah user manual, printed pages 9–14: handling, charging, maintenance and specifications. Official 20 Ah product listing.

## E2 TATTU TA7200Pro charger

Selected from the chargers listed in E1. Match the manufacturer lead to the actual battery connector; facilities staff approve the supply circuit and operating power.

TA7200Pro user manual, printed pages 5–9: precautions, display, connection and pause procedure. Manufacturer support page provides current manuals. Demonstrate the automatic start behaviour before permitting student participation.

## E3 Bambu Lab P2S printer

P2S user manual, version 1.0, May 2026: safety, first print, part removal and Bambu Studio. P2S support wiki.

The training job uses Quiver 2313 battery connector PCB cover. In the engineering repository, find:

•  CAD: src/quiver/supporting_structure/equipment_mount/steps/2313_bc_pcb_cover.step

•  Mesh: docs/Manufacturing/Assembly-Guides/assets/models/structural/2313.stl

Use the instructor-released revision. The BOM specifies PETG-CF; nozzle, filament, plate and print settings require approval. A successful print is not aircraft acceptance.

## Demonstration tray

Prepare labelled examples of 2111/2131 attachment spacers, 3112/3122 propellers, HAR-0008 signal harness and the 2313 cover. Demonstrate Pix32 V6 identification on the isolated avionics assembly. Use photographs for damage scenarios; do not create faults in flight hardware.

These are the selected course examples, not an assertion that every item has already been purchased by the university. Record actual asset IDs and authorisations in the equipment register.